PPPoE Disconnect Issue

Copyright © 2005 SerComm Corporation. All Rights Reserved. SerComm Corporation reserves the right to make changes to this document without notice. SerComm Corporation makes no warranty, representation or guarantee regarding the suitability of its products for any particular purpose. SerComm Corporation assumes no liability arising out of the application or use of any product or circuit. SerComm Corporation specifically disclaims any and all liability, including without limitation consequential or incidental damages; neither does it convey any license under its patent rights, nor the rights of others.

- Problem description:

Description from Customer:

Internet connection disconnects every 5~10 minutes.

WAN mode: PPPoE

DSLAM: Huawei UA5000

Sercomm’s description on this issue after analysis:

    - PPPoE disconnect because not receive PPP LCP response after 5~10 minutes

    - DSL driver can’t receive packet after 5~10 minutes

- Root cause

This issue is caused by 3 factors:

  - Network enviroment

In customer’s network there are many PPPoE session packet(PADI,PADT...) that should not be send to CPE. It’s caused by improper network architecture.

  - DSL driver

Broadcom DSL driver preallocate buffer for receiving packet from DSL(120 recv buffer). When this issue happens, DSL driver can’t get free receive buffer queue. Upper layer can’t receive any packet from DSL.

This issue also happens on Ethernet WAN if Ethernet drive preallocate buffer in the same way. In Broadcom platform the Ethernet recv buffer is 200. So it need about 10~20 minutes to reporduce this issue in Ethernet WAN mode.

  - PPPoE daemon

PPPoE daemon dosn’t close socket for PPPoE session after the PPPoE connection established. This socket keeps open for sending PADT when session disconnect. After PPPoE session is established, nobody will read the packet from receive buffer of this socket. But there are many unexpected PPPoE session packets are sent to CPE in customer’s network enviroment. This causes the PPPoE session packet to be queued on the socket receive buffer and will not released until PPPoE daemon is down. There is  a limitation for the socket receive buffer length named SO_RCVBUF length. The default value is 134144 bytes. Because the packet size of PPPoE session packet is small, it will use all DSL driver preallocated receive buffer before reachs SO_RCVBUF length. That’s why PPP can’t receive any packet from DSL driver and PPPoE session can be established successfully again after disconnect and reconnect. Please refer to the flow chart blow:

- Solution:

  - Close the PPPoE socket after PPPoE session is established. Reopen when PPPoE want to send PADT. Attached the source before and after modified.

  - How can I know if my code has this bug?

All Linux base model that use PPPoE plugin patch4 should have this bug.

- Test:

  - You can verify this issue by a packet generate tool named bittwist. Here are the steps to reproduce and verify this issue:

a. Connect the PC installed bittwist on the WAN side of device and device is in PPPoE mode connected

b. Run "bittwist -d"” in command line to get the interface that we used to send packets

In my pc, the interface is 4.

c. Run” bittwist -i 4 -s -1 -m 0 -p 1 D:\bittwise\pppoe.pcap” in command line. Here ’-i 4’ define the interface to send out the packets. The file pppoe.pcap is packet trace caputred by wireshark. I attached a sample for testing.

d. PPPoE will disconnect after a while because can’t receive any packet from WAN.

2. Retest this case after integrated the patch. PPPoE will not disconnect.